# NON-NUCLEAR 
TEST RIG DETERMINATION AND INSPECTION CHECK LIST

References:	(a)	NAVSEA S9086-RJ-STM-010 - NSTM Chapter 504 (Pressure, Temperature and Other Mechanical and Electromechanical Measuring Instruments)

(b)	NAVSEA S9505-AF-MMA-010 - Piping Systems/Submarine Non-Nuclear Piping Systems Test Manual

(c)	NAVSEA S9086-RK-STM-010 - NSTM Chapter 505 (Piping Systems)

| ITEM NO. | ATTRIBUTE | CHECK OFF or N/A |
| --- | --- | --- |
| 1. Complete the following based on information provided by the requesting activity. |  |  |
| a. | System or component to be tested. |  |
| b. | Test Pressure Reference (If Item Number) |  |
| c. | Number of test gages required: Total__________ Primary___________ Backup __ Indicate zero if an installed system gage will be used. |  |
| d. | Level of cleanliness required_________________ |  |
| e. | Planned method of overpressure protection (e.g., manual using installed system valve plus one automatic relief included with test rig, or other combination). Unless otherwise specified, the rig will be provided with one manual and one automatic relief. |  |
| 2. GAGES: |  |  |
| NOTE: TYPICAL GAGES FOR MOST NON-NUCLEAR TESTS ARE LISTED IN REFERENCE (a), TABLE 504-6-1. |  |  |
| CAUTION: REFERENCE (a) TABLE 504-6-1 ALLOWS EXCEPTIONS TO THE GAGES LISTED IN THE TABLE. IF USING A GAGE NOT LISTED ON THE TABLE, IT MUST STILL COMPLY WITH THE CRITERIA SPECIFIED IN THIS APPENDIX. |  |  |
| a. | ANALOG GAGES The pressure gage range based on the maximum test pressure up to and including the relief valve setting must be: - greater than the test pressure and - not more than 200% of the maximum test pressure. |  |
|  | For tests 0-60 psi and below, the range may exceed 200% of the test pressure but will be as low as practical. |  |
|  | Test gage pressure graduations are equal to or smaller than those specified by reference (a) Table 504-6-1. |  |
|  | Gage accuracy must be at least + or - 1.0 % of the gage span. |  |
|  | Primary gage accuracy must be at least + or - 1.0% of the gage span. |  |
| b. | DIGITAL GAGES Digital gages are authorized for use by reference (b). Digital gages of any range may be used as long as the gage accuracy is equal to or better than + or - 1.0% of the gage span. Gage range may exceed 200 % of the test pressure as long as the gage accuracy is equal to or better than + or - 1.0% of the test pressure. Gage accuracy must be equal to or better than + or - 1.0% of the test pressure. |  |
| c. | RECORD GAGE DATA: Primary gage range psi Cal due date Ser Number________________ Backup gage range psi Cal due date Ser Number |  |
| d. | Gages have been calibrated within the required periodicity. Circle applicable item: (1) Normal metrology requirements list calibration frequency (This is the frequency unless specified otherwise for the specific test). (2) Special frequency required by (Indicate required frequency and applicable reference). |  |
| 3. OVERPRESSURE PROTECTION: |  |  |
| a. | The manual release of pressure is the primary method of protection. |  |
| b. | Backup relief valve, automatic or manual, will be provided. (Automatic preferred unless otherwise specified). |  |
| c. | If the temporary relief valves provided are to be installed into the system, installation must be such that it does not damage the system or system components. Review reference (b) or (c), as applicable. |  |
| d. | The over-capacity of relief valves should not cause excessive thermal and pressure shock. (Relief valve capacity should not be grossly higher than pressure source capacity). |  |
| e. | The blow down characteristics must be compatible with the system being tested. (The reseating characteristics of automatic reliefs should be consistent with the requirements of the system being tested). (Review manufacturer's data). |  |
| f. | Valve accumulation at the flow of the test pressure source must not exceed 10% of the relief valve set point. |  |
| g. | Temporary automatic relief valves which have more than one inlet and therefore provide different relief valve performance characteristics (e.g., set pressure) must not be used. |  |
| h. | Temporary automatic reliefs which have a rapid adjustment feature by which the set point may be inadvertently changed must not be used. |  |

| ITEM NO. | ATTRIBUTE | CHECK OFF or N/A |
| --- | --- | --- |
| 4. OVERPRESSURE PROTECTION SETPOINT DETERMINATION: |  |  |
| a. | If performing a maximum operating pressure test, verify that installed system relief valves are unisolated and not blocked or gagged. The set point for the manual release of pressure must be based on installed system relief set points. |  |
| CAUTION: WHEN PERFORMING MAXIMUM OPERATING PRESSURE TESTS, INSTALLED RELIEF VALVES PROVIDE SYSTEM PROTECTION. SYSTEM RELIEF VALVES MUST NOT BE REMOVED FROM SERVICE. |  |  |
| b. | Record required hydrostatic test pressure psi. |  |
| c. | Record required overpressure protection set point psi. Circle applicable determination method: the lesser of: (1) 100 psi over test pressure. (2) 10% above test pressure. |  |
| 5. TEMPORARY EQUIPMENT REQUIREMENTS: |  |  |
| a. | Temporary connections and equipment (cap, spool pieces, jumpers, blank flanges, hoses, etc.) must be verified to be capable of withstanding the test pressure. |  |
| b. | Verify that within the last year and subsequent to disassembly or replacement of any test rig equipment the assembled test rig has been satisfactorily tested to a pressure approximately 15% above the maximum pressure it will witness during the system hydrostatic test. (Hydro is not required to test pressure gage and relief valve replacement or fittings between subassemblies designed to be broken for portability. Set point testing of the test rig relief valve while on the test rig meets this requirement). |  |
| c. | Verify the rig has been cleaned to meet the system cleanliness requirements of the system being tested. |  |
| d. | No quick-disconnects with check features are to be used in the test rig. |  |
| 6. OPERATIONAL TEST: |  |  |
| INSTRUCTIONS: The following checks must be performed with the test rig isolated from the system being tested. This may be done by blanking the hose prior to connecting the rig to the system. References (b) and (c) provide a sample step-by-step procedure for these checks. Equipment provided by an FMA or Industrial Activity must be tested prior to being issued to a tended ship. If documentation is provided by the issuing FMA or Industrial Activity that the assembled test rig has been verified to meet all of the requirements of this appendix, the operational test need not be repeated by the end user. NOTE: FOR SHOP TEST FACILITIES (I.E., INTERMEDIATE MAINTENANCE ACTIVITIES, NAVAL SHIPYARDS AND TENDERS) WHERE HYDROSTATIC TESTS ARE CONDUCTED AT LEAST ONCE EVERY TWO WEEKS AND WHERE THE TEST GAGES (BACKUP AND MASTER) ARE COVERED UNDER THE NAVY METROLOGY AND CALIBRATION (METCAL) SYSTEM, CROSS CHECKING THE GAGES BEFORE TEST IS NOT REQUIRED. |  |  |
| a. | ANALOG GAGES: Mark all test gages with either a red grease pencil, red tape, or the use of an integral red hand or other suitable mark at the required relief over pressure protection set point, half way between the test pressure and the relief valve set point. |  |

| ITEM NO. | ATTRIBUTE | CHECK OFF or N/A |
| --- | --- | --- |
|  | DIGITAL GAGES: Digital gages must have the manual over-pressure value (red hand setting) posted in the immediate vicinity of the display with red digits approximately the same size as the digits in the instrument display. For example, red hand settings may be marked on a blank label plate of “Whiteboard” material affixed to the test rig and the setting entered using a non-permanent red marker (dry erase). |  |
| b. | Raise rig pressure to the specified test pressure. Verify primary and backup gages are in agreement. Maximum difference between the two gage readings after height correction is no more than 2%. Calculate 2% for each gage using the full-scale pressure measured by each gage (i.e., a 0-100 psi gage, 2% = 2 psig, the maximum difference between two gages with a 0-100 psi scale at a given pressure is 2 psig). Required accuracy + or - psig. If not in agreement, replace or re-calibrate and repeat this step until satisfied. |  |
| c. | Continue to raise pressure to the specified overpressure protection set point. If the manual relief is installed on the rig (vice using an installed system valve), with the pressure source operating and at the specified manual relief set point, verify that the manual relief and associated piping relieve the capacity of the pressure source. |  |
| d. | Set or confirm setting of the automatic relief valve. With the pressure source operating and at the relief set point, verify that the automatic relief and associated piping relieve the capacity of the pressure source. |  |
| e. | Vent the rig to reduce pressure to reseat the automatic relief. Slowly raise pressure again to recheck the automatic relief set point. |  |

Completed by ___________________Items Completed ______________Date ______________

Completed by ___________________Items Completed ______________Date ______________

Reviewed (Div Off or EDO) ___________________________________Date _______________